Heatflow smoker change from 1 component to 2 components

Here are the changes needed to the smoker.data file:

1. Option kmass now extends from 0-4. Set kmass=4 to simulate heat, and two components c1 and c2.

2. If kmass=4, then you need a second set of boundary conditions for mass transport, immediately after those for c1.

3. Regardless of kmass, (i.e. even with just 1 component) two retardation terms and two decay terms are needed on the line with the dispersivities, one for c1 the other for c2:

i.e. ...  retardc1,retardc2,decayc1,decayc2 ...

(remember the decay terms are in seconds^-1). If you only have component c1, then just enter '0' for the c2 values.

4. The initial condition for the mass components c1 and c2 must now include the component number like this:

1  11  1 2  1  11  1  00.0  00.     +1       ;CONC.I.C.(END WITH -1) ix1-ix2,iy1-iy2,iz1-iz2,kcomp=1,conc_bot, conc_top, more.

1  11  1 2  1  11  2  00.0  00.     -1        ;CONC.I.C.(END WITH -1) ix1-ix2,iy1-iy2,iz1-iz2,kcomp=2,conc_bot, conc_top, more.

where kcomp is 1 or 2 for component c1 and c2, respectively. If c2 doesn't exist, then you would just have lines for component number 1.